C.O.I.S.A

Code fOr calculatIon of uSing a drivetrAin

Variable Reference Manual — Brushed DC Motors

Liga Acadêmica de Robótica e Inovação — LARI / Unifor

Line Follower Category

# 1. Motor Parameters

These variables describe the electrical and mechanical characteristics of the drive motor. Fill them in from your motor datasheet before running the calculator.

## 1.1  MOTOR_KV

RPM per volt at no load. This is the motor's speed constant — it tells you how fast the motor spins for every volt applied, assuming no load on the shaft.

| Motor | Typical KV Range | Notes |
| --- | --- | --- |
| 1020 Coreless | 8,000 – 18,000 rpm/V | Most common in Brazilian line follower robots |
| 720 Coreless | 10,000 – 20,000 rpm/V | Smaller, higher KV, less torque |
| 816 Brushed | 6,000 – 12,000 rpm/V | Good torque, slightly heavier |
| N20 Gearmotor | Built-in reduction | KV refers to internal motor; use output RPM instead |
| 130 Toy Motor | 5,000 – 10,000 rpm/V | Low cost, high variance — measure yours |

ℹ  If your datasheet gives no-load RPM at a specific voltage, KV = RPM ÷ Voltage.

⚠  Higher KV means more speed but less torque at the same voltage. Match KV to your gear reduction.

## 1.2  MOTOR_VOLTAGE

The voltage supplied to the motor by your battery pack. Use the nominal voltage, not the fully-charged peak.

| Battery Type | Cells | Nominal Voltage | Fully Charged |
| --- | --- | --- | --- |
| LiPo 1S | 1 | 3.7 V | 4.2 V |
| LiPo 2S | 2 | 7.4 V | 8.4 V |
| NiMH 3-cell | 3 | 3.6 V | 4.2 V |
| NiMH 4-cell | 4 | 4.8 V | 5.6 V |
| Li-ion 18650 1S | 1 | 3.6 V | 4.2 V |

✔  Use 3.7 V for a 1S LiPo — the calculator will correctly give you max RPM at 100% PWM.

⚠  Never enter 4.2 V (fully charged). The calculator assumes nominal voltage for consistent results across a race run.

## 1.3  MOTOR_STALL_TORQUE

Maximum torque the motor produces when the shaft is held completely still (zero RPM). This is the peak mechanical output — used to calculate available wheel torque through the gear reduction.

| Motor | Stall Torque (mNm) | Source |
| --- | --- | --- |
| 1020 Coreless | 0.8 – 1.5 | Datasheet / RC hobby forums |
| 720 Coreless | 0.4 – 0.9 | Datasheet |
| 816 Brushed | 1.2 – 2.5 | Datasheet |
| N20 at 3V | 0.3 – 0.8 (motor) | Post-gearbox torque is much higher |
| 130 Motor | 0.5 – 1.0 | Measure — high unit-to-unit variation |

ℹ  If your datasheet gives stall torque in g·cm, multiply by 0.0981 to convert to mNm.

ℹ  The calculator uses this value at 100% PWM and scales linearly — 50% PWM gives 50% stall torque.

## 1.4  MOTOR_STALL_CURRENT

Current drawn when the motor is stalled. Used to estimate total current draw from the battery at each PWM level. Important for battery selection and ESC/driver rating.

| Motor | Stall Current (A) | Notes |
| --- | --- | --- |
| 1020 Coreless | 1.5 – 2.5 A | Check your specific motor — varies by winding |
| 720 Coreless | 0.8 – 1.5 A |  |
| 816 Brushed | 2.0 – 4.0 A | Higher current — ensure driver can handle it |
| N20 Gearmotor | 0.3 – 0.6 A | Low current; easy on batteries |
| 130 Motor | 1.0 – 2.5 A | Measure directly for accuracy |

⚠  Your motor driver must be rated above MOTOR_STALL_CURRENT × NUM_MOTORS. Exceeding this rating will burn the driver.

✔  The calculator shows current at each PWM point — use 75% PWM values for a realistic operating estimate.

# 2. Gear Setup Parameters

These variables define the physical geometry of your spur gear drivetrain. They control the speed-torque trade-off and must match your actual printed or purchased gears.

## 2.1  MY_PINION_TEETH / MY_WHEEL_TEETH

The tooth counts on each gear. The ratio is Wheel ÷ Pinion — a higher ratio means more torque and less speed.

| Ratio Range | Effect | Best for |
| --- | --- | --- |
| 3:1 – 5:1 | High speed, low torque | Lightweight robots on smooth tracks |
| 5:1 – 8:1 | Balanced (recommended range) | Most line followers — good all-round |
| 8:1 – 12:1 | High torque, moderate speed | Heavier robots or grippy floors |
| > 12:1 | Very high torque, low speed | Obstacle course, not speed runs |

ℹ  The Ideal Ratio Finder scans all combinations in PINION_RANGE and WHEEL_RANGE and scores them automatically.

✔  Hunting tooth rule: GCD(pinion, wheel) must equal 1 so every tooth pair eventually contacts — this is enforced by the calculator.

## 2.2  GEAR_MODULE

Module defines tooth size. Two gears must have the same module to mesh. Smaller module = more teeth fit in a smaller diameter = finer, more fragile teeth.

| Module | Tooth Size | Recommended Use | Min Printable Wall |
| --- | --- | --- | --- |
| 0.3 | Very fine | 1020/720 motors — standard for line followers | FDM: marginal. Resin: good |
| 0.4 | Fine | 816/130 motors — slightly more robust | FDM: acceptable |
| 0.5 | Medium | N20 and larger motors | FDM: reliable |
| 0.8 | Coarse | High-torque robots, large wheels | FDM: easy |
| 1.0 | Very coarse | Heavy-duty or slow robots | FDM: easy |

⚠  Center distance (CAD) = Module × (Pinion + Wheel) ÷ 2. This is the shaft-to-shaft distance in your chassis — measure and match it exactly.

## 2.3  GEAR_EFFICIENCY

Fraction of motor power that actually reaches the wheel after accounting for friction in the gear mesh. Always less than 1.0.

| Condition | Typical Efficiency |
| --- | --- |
| Well-lubricated spur gears | 0.90 – 0.95 |
| Dry spur gears (typical FDM print) | 0.82 – 0.90 |
| Poorly aligned or rough teeth | 0.70 – 0.82 |
| Worm gear stage (if used) | 0.40 – 0.70 |

✔  Use 0.85 as a safe default for 3D-printed spur gears without lubrication. Add a tiny drop of light machine oil to reach 0.90+.

# 3. Wheel Parameters

## 3.1  WHEEL_DIAMETER_MM

The rolling diameter of the wheel in millimetres. This determines how far the robot travels per wheel revolution and directly affects top speed.

| Setting | Behaviour |
| --- | --- |
| None (default) | Wheel diameter is calculated automatically as Module × Wheel Teeth. Use this when the wheel IS the gear (teeth on the rim). |
| Enter a value (e.g. 25) | Use the actual measured diameter. Use this when the wheel is separate from the gear (hub + tyre assembly). |

ℹ  Pitch diameter (Module × Teeth) is the theoretical rolling diameter for a gear. If your wheel has a tyre added on top of the gear, measure the tyre OD and enter that value.

⚠  Getting this wrong will make all speed calculations incorrect. When in doubt, measure the wheel with a calliper.

# 4. Robot Parameters

## 4.1  ROBOT_MASS_G

Total mass of the robot in grams, including battery, all electronics, and chassis. Weigh it fully assembled on a digital scale.

| Class / Build Style | Typical Mass Range |
| --- | --- |
| Ultra-light FDM (1S LiPo) | 40 – 70 g |
| Standard FDM line follower | 70 – 120 g |
| With sensors + Arduino Nano | 90 – 140 g |
| Metal chassis | 150 – 250 g |

⚠  Never guess your mass. A 20 g error changes the required torque calculation significantly and can cause the calculator to give a GOOD rating when the real robot barely moves.

## 4.2  FRICTION_COEFF

The coefficient of friction between your wheel surface and the competition floor. This limits how much force the wheel can exert before spinning in place (slip).

| Wheel / Floor Combination | Friction Coeff (μ) | Notes |
| --- | --- | --- |
| Silicone tyre on smooth MDF | 0.60 – 0.80 | Excellent grip — common in high-end builds |
| Rubber O-ring on MDF | 0.45 – 0.65 | Good standard choice |
| Rubber band on MDF | 0.40 – 0.60 | Works well, degrades over time |
| Printed PLA/ABS wheel on MDF | 0.25 – 0.40 | Poor grip — avoid if possible |
| Printed nylon gear as wheel | 0.20 – 0.35 | Very low — gear teeth on floor |
| Smooth polycarbonate on MDF | 0.20 – 0.30 | Lowest grip — not recommended |

✔  Test your actual setup: place the robot on the track, attach a spring scale horizontally, and pull until the wheel slips. μ = Pull Force (N) ÷ Robot Weight (N).

## 4.3  NUM_DRIVE_WHEELS

Total number of wheels in contact with the floor that are driven by motors. Used to calculate the torque requirement per wheel.

| Configuration | NUM_DRIVE_WHEELS | Notes |
| --- | --- | --- |
| 2-wheel differential | 2 | Classic line follower — 2 rear drive, 1 front caster |
| 4-wheel drive | 4 | All four wheels powered — better grip distribution |
| 4-wheel, 2 driven | 2 | Only rear pair driven — set to 2 even with 4 wheels |

## 4.4  NUM_MOTORS

Total number of motors in the drivetrain. Used to calculate total current draw from the battery.

| Setup | NUM_MOTORS |
| --- | --- |
| 1 motor per side (standard differential) | 2 |
| 1 motor per wheel (4WD independent) | 4 |
| Single motor + differential gearbox | 1 |

ℹ  NUM_MOTORS and NUM_DRIVE_WHEELS can differ. Example: 4 wheels, 2 motors (each motor drives 2 wheels via an axle).

# 5. Ratio Search Parameters

These variables control the Ideal Ratio Finder — the part of COISA that scans all gear combinations and ranks them by score.

## 5.1  TARGET_SPEED_MS

If you have a specific target speed in m/s, enter it here. The scorer will reward combinations that land close to this speed at 50% PWM. Set to None for automatic torque-based scoring.

| Value | Behaviour |
| --- | --- |
| None | No speed target — scorer rewards any speed between 0.8 and 5.0 m/s |
| 1.5 | Target 1.5 m/s at 50% PWM — good for tight/technical tracks |
| 2.5 | Target 2.5 m/s at 50% PWM — typical competition speed |
| 4.0 | Target 4.0 m/s at 50% PWM — high-speed track, experienced team |

✔  Set TARGET_SPEED_MS to your competition target and let the finder pick the gear ratio — then verify the torque margin is GOOD or better.

## 5.2  PWM_POINTS

List of PWM percentages to evaluate and print in the main results table. These correspond to motor driver duty cycle values.

| PWM % | Arduino analogWrite | Use case |
| --- | --- | --- |
| 25% | ~64 | Low-speed test / sensitive tracks |
| 50% | ~128 | Typical PID base speed |
| 75% | ~192 | Fast straight sections |
| 100% | 255 | Full throttle / burst acceleration |

## 5.3  PINION_RANGE / WHEEL_RANGE

The search space for the Ideal Ratio Finder. Wider ranges take longer but find more options.

| Variable | Default | Recommendation |
| --- | --- | --- |
| PINION_RANGE | range(8, 16) | Keep small — large pinions are physically too big for micro motors |
| WHEEL_RANGE | range(30, 91) | Adjust upper limit to your chassis space (MAX_WHEEL_DIAM_MM controls this too) |

## 5.4  MIN_CENTER_DIST_MM / MAX_WHEEL_DIAM_MM

Physical constraints that filter out mechanically impossible combinations.

| Variable | Default | What it filters |
| --- | --- | --- |
| MIN_CENTER_DIST_MM | 7.0 mm | Rejects gear pairs whose shaft-to-shaft distance is too small to print reliably |
| MAX_WHEEL_DIAM_MM | 40.0 mm | Rejects wheels larger than your chassis allows |

✔  Measure your chassis: the maximum distance from motor shaft to wheel axle is your practical center distance limit. Set MIN_CENTER_DIST_MM to your motor-shaft-to-chassis-edge clearance.

# 6. Quick Reference — All Variables

| Variable | Unit | Typical Range | What to do if unsure |
| --- | --- | --- | --- |
| MOTOR_KV | rpm/V | 8,000 – 20,000 | Check datasheet; measure no-load RPM ÷ voltage |
| MOTOR_VOLTAGE | V | 3.7 – 7.4 | Use battery nominal voltage (3.7 for 1S LiPo) |
| MOTOR_STALL_TORQUE | mNm | 0.8 – 2.5 | Check datasheet; 1.2 is a safe default for 1020 |
| MOTOR_STALL_CURRENT | A | 0.8 – 4.0 | Check datasheet; 1.8 is typical for 1020 |
| MY_PINION_TEETH | teeth | 8 – 15 | Count your gear teeth or check your CAD file |
| MY_WHEEL_TEETH | teeth | 30 – 90 | Count your gear teeth or check your CAD file |
| GEAR_MODULE | mm | 0.3 – 1.0 | Match the module engraved or specified in your gear file |
| GEAR_EFFICIENCY | — | 0.80 – 0.92 | Use 0.85 for dry 3D-printed spur gears |
| WHEEL_DIAMETER_MM | mm | 15 – 40 or None | None if wheel = gear; measure OD if separate tyre |
| ROBOT_MASS_G | g | 50 – 200 | Weigh fully assembled with battery |
| FRICTION_COEFF | — | 0.20 – 0.80 | Use 0.45 for rubber on smooth floor |
| NUM_DRIVE_WHEELS | — | 2 or 4 | Count wheels touching the floor and driven |
| NUM_MOTORS | — | 1 – 4 | Count motors; usually 2 for differential |
| TARGET_SPEED_MS | m/s | 1.0 – 5.0 / None | None for automatic; enter target for speed-tuned search |
| MIN_CENTER_DIST_MM | mm | 5.0 – 10.0 | Measure minimum printable shaft gap in your chassis |
| MAX_WHEEL_DIAM_MM | mm | 20 – 50 | Measure maximum wheel space in your chassis |

# 7. Interpreting the Output

## 7.1  Torque Margin

The most important output metric. It tells you how much torque headroom you have above the minimum needed to move the robot.

| Rating | Margin | Meaning |
| --- | --- | --- |
| EXCELLENT | ≥ 40% | Plenty of headroom for acceleration, cornering, and sensor-weight additions |
| GOOD | 25 – 39% | Acceptable — robot will perform reliably under normal conditions |
| MARGINAL | 10 – 24% | Robot moves but may struggle with cornering loads or additional weight |
| WEAK | 0 – 9% | Very little headroom — unreliable, especially at low PWM |
| FAIL | < 0% | Motor cannot overcome friction — robot will not move at this PWM level |

## 7.2  Slip Warning

If the Slip? column shows YES, the drive force exceeds the friction limit — the wheel will spin without moving the robot forward. This wastes power and reduces control.

✔  Slip at high PWM is normal and manageable with PID. Slip at 25% PWM is a problem — increase friction (better wheel material) or reduce gear ratio.

## 7.3  Score (0 – 100)

A composite score computed at 50% PWM. It weights torque margin (40 pts), speed range (35 pts), current draw (15 pts), and printability of center distance (10 pts).

ℹ  Two ratios with the same score are equally good — choose the one with the center distance that fits your chassis best.